Claw Mechanism

- Claw Segment:

- Buy more spring hinges (1-2 more packs?) (Purchase) (bought in locker)

- Buy elastic wrist straps as sent by Fern (Purchase) (bought in locker)

- Check M3 bolts fit tomorrow with parts, potentially order more with self-tapping inserts. (dependence on parts arriving)

- (Gather all changes needed when reviewed tomorrow and adjust for the next print – as little reprints as possible to conserve filament and print) ^

- Need Spring Hinge CAD made to view an assembly (no dependence)

- Forearm attachment methods

  - Settle on forearm length to design for (quick)

  - Need to decide clamp designs and how it hooks onto arm bands (tabbed base with the clamp on top? (1. Bowden Clamp on wristplate/end of segment, 2. Bowden Clamp on its separate piece)

    - Need a snap clamp CAD made and printed for testing

    - Need to print the screw clamp

    - Nice to test print the screw clamp and the snap clamp to evaluate them each piece.

    - Make a reciprocal clamp part?

  - Decide Hook design top (carabiner? Just simply hooking onto strap may not be secure enough) - see Varun’s design

eNable finger for left hand digit

- Resized sockets – check these and for print orientation

- For left digit - @ 200% scale, M5 x 3 set screws, 4mm aluminium wire needed. (Purchase)

- Also need 3mm -3.5mm – 4mm drill bits to drill out holes. (Purchase)

Cable-Pulley Mechanism

- Capstan:

  - Add grooves down the side of capstan bed to better fit snap lid (snap lid function on solidworks)

  - Resize Capstan bed complementary hole diameter to ~+0.5mm over capstan diameter.

  - Embed 1xCapstan bed (large) into backplate

Right Hand:

- Review hook and channel size when Harshad prints by Monday

- Decide Bowden sheath run path.

- Fabric choice underneath backplate, “mattress cover” sheet underneath or silicon pads to create an air gap

Left Hand:

- Reshape backplate to left hand contour

- Adjust to only 1 hinge attachment point