Meeting 15th  June 2026 9am.

Attendees: Alex, Mervyn

Mervyn

(Left Hand)

String pulley -

Thumb contracts, pulls string that closes middle fingers/middle finger apparatus up top. Pinky contraction eases tension in the string, loosening grip.

(Need to figure out string pulley configuration)

Magnetic wrist strap for attachment (see slap wrist strap bands)

Magnetic ENDS of wristband attached.

https://wrapitstorage.com/pages/magsnap

see his page in Google Doc for more details.

Nylon rings for his digits?

Any flexible filament material. Flexible TPU? To act as a wrist strap?

Fabric choice? Not sweaty irritable, biocompatible.

Would not encourage ABS, very stiff. Usually TPU, PLA.

Velcro can be irritating.

Write to Eito about any fasteners, Leg Team  Electric and Knee Team (3D printing), ankle materials.

Likely not carbon fibre due to restrictions/complications in cutting, forming, requiring a mold to produce in the shape of his hand.

PC filament, more translucent than ABS but almost as tough.

Potentially, wrist strap goes through a slot in the 3d printed prosthetic hand, going around the wrist and securing it.

Actions

- Advised Mervyn to realise the string/pulley configuration, and establish the wrist/device attachment method. We’ll discuss his design late Wednesday, OR Thursday morning and judge whether to carry it to manufacturing or drop for other designs.